CPE301 – SPRING 2019

# Design Assignment 4A

Student Name: Cody Hudson

Student #: 8000603642

Student Email: hudsoc1@unlv.nevada.edu

Primary Github address: https://github.com/eed911/class_proj.git

Directory:

https://github.com/eed911/class_proj/tree/master/DesignAssignments/DA4A/Project_4A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of Components used

- ATmega328P Xplained

- Switch

- Potentiometer

- Motor Driver

- DC Motor

Block diagram with pins used in the Atmega328P

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

1. Write a C AVR program that will monitor the LM34/35 connected to an Analog pin

(PC5) to display the temperature in F on the serial terminal every 1 sec. Use a timer

with interrupt for the 1 sec delay. Use a FTDI chip for serial to USB conversion.

2. Use the ATMEL Studio Data Visualizer or any Charting program to display the values

in time

Code:

/*

* DA4A.c

*

* Created: 4/11/2019 11:35:52 AM

* Author : hudsoc1

*/

#define F_CPU 16000000UL

#include <avr/io.h>

#include <stdio.h>

#include <util/delay.h>

#include <avr/interrupt.h>

// Define Baudrate for UART

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

// Function Declarations

void read_adc(void);

void adc_init(void);

void USART_init( unsigned int ubrr );

void USART_tx_string( char *data );

volatile unsigned int adc_motor_speed;

char outs[20];

int main(void)

{

adc_init();             // Initialize the ADC

USART_init(BAUD_PRESCALLER);  // Initialize the USART

USART_tx_string("Connected!\r\n");    // we're alive!

_delay_ms(125);         // wait a bit

DDRD = 0xFF; //DDRD = (1<<3); // Set Port D as an output direction THIS IS REQUIRED TO OUTPUT THE PWM

// In the next line of code, we:

// 1. Set the compare output mode to clear OC2A and OC2B on compare match.

//    To achieve this, we set bits COM2A1 and COM2B1 to high.

// 2. Set the waveform generation mode to fast PWM (mode 3 in datasheet).

//    To achieve this, we set bits WGM21 and WGM20 to high.

TCCR2A = _BV(COM2A1) | _BV(COM2B1) | _BV(WGM21) | _BV(WGM20);

// In the next line of code, we:

// 1. Set the waveform generation mode to fast PWM mode 7 —reset counter on

//    OCR2A value instead of the default 255. To achieve this, we set bit

//    WGM22 to high.

// 2. Set the prescaler divisor to 1, so that our counter will be fed with

//    the clock's full frequency (16MHz). To achieve this, we set CS20 to

//    high (and keep CS21 and CS22 to low by not setting them).

TCCR2B = _BV(WGM22) | _BV(CS20) | _BV(CS21) | _BV(CS22);

// OCR2A holds the top value of our counter, so it acts as a divisor to the

// clock. When our counter reaches this, it resets.

OCR2A = 255;

// This is the duty cycle. Think of it as the last value of the counter our

// output will remain high for. Can't be greater than OCR2A of course.

OCR2B = 0;

PCMSK1 |= (1<<PCINT10);

PCICR |= (1<<PCIE1);

sei();

while (1)

{

read_adc();

_delay_ms(250); // Why delay?

adc_motor_speed = adc_motor_speed/4;

snprintf(outs,sizeof(outs),"%3d\r\n", adc_motor_speed);  // print it

USART_tx_string(outs); // Print random integer number

OCR2B = adc_motor_speed; // Replace with adc value

}

}

/* INIT ADC */

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(0<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC0 (PC0 PIN27)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i = 4;

adc_motor_speed = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_motor_speed+= ADC;

_delay_ms(50);

}

adc_motor_speed = adc_motor_speed / 4;  // Average a few samples

}

/* INIT USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);

UBRR0L = (unsigned char)ubrr;

UCSR0B = (1 << TXEN0);     // Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);    //asynchronous 8 N 1

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data )

{

while ((*data != '\0'))

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;

data++;

}

}

ISR(PCINT1_vect){

DDRD ^= 0xFF;

_delay_ms(1000); //Ground Bounce

char check = "Pin Change ";

USART_tx_string(check); // Print random integer number

// Check if I bit is set in IREG

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Screenshot of demo:

- VIDEO LINKS OF EACH DEMO

Demo1:

https://youtu.be/qg6q-xfl8Aw

- GITHUB LINK OF THIS DA

https://github.com/eed911/class_proj.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Cody Hudson